[image: image1.png]v #186
MATERIAL SAFETY DATA SHEET
NFPA RATING: Health= 0 Flammability= 2 Reactivity= 0
“MIS RATING: Health= 0 Flammability= 2 Reactivity= 0

SECTION I - IDENTITY AND MANUFACTURER'S INFORMATION (0655A)

Manufacturer's Name: HILLYARD INDUSTRIES Product Name: LEMON ODOR COUNTERACTANT
Address: 302 North Fourth Street Date Prepared: June 22, 1994
St. Joseph, MO 64501 Prepared by: Regulatory Affairs Department

Emergency Telephone No.: (800)-424-9300 (Only in the event of chemical emergency involving a spill, leak, fire,
exposure or accident involving chemicals.) Other information calls: (816)233-1321(Ext. 303)
SECTION II — INGREDIENTS/IDENTITY INFORMATION
Components ACGIH OTHER LIMITS
(Specitic Chemical Identity: CAS# OSHA TLV RECOMMENDED Yo
Common Name(s) PEL
Isopropy! alcohol (1) (not strong acid process) 67-63-0 400ppm 400 ppm N/A 2-6
Water 7732-18-5 N/A N/A N/A -
Qctyl phenoxy polyethoxy ethanol 9036-19-5 N/A N/A N/A -
Lemon perfume ’ mixture N/A N/A N/A -—

unknown to

Hillyard

(1) Regulated by OSHA and the following states: CT, FL, IL, LA, MA, NJ, PA, RI

SECTION HI ~ PHYSICAL / CHEMICAL CHARACTERISTICS

Boiling Point: corr. 211°F Specific Gravity (H20 = 1): 25°C= 0981 &39°C= 0984
Vapor Pressure (mm Hg.): 17.9 Percent Volatile by Volume (%): average 98.2

Vapor Density (AIR=1): 0.7 Evaporation Rate (ethyl ether =1):  slower than 1

Solubility in Water : complete Appearance and Odor: light amber liquid; lemon odor

pH (concentrate) = 7.19 average

SECTION 1V — FIRE AND EXPLOSION HAZARD DATA
Flash point: 122°F (Tag Closed Cup) corrected ~ Flammable Limits: LEL= 1.9% UEL= N/A (mixture)
Extinguishing Media: Water spray, carbon dioxide, dry foam oo
Special Fire Fighting Procedures: Respiratory protection required for fire fighting personnel. Use water spray to
cool fire exposed surfaces and to protect personnel.
Unausual Fire and Explosion Hazards: None known to Hillyard Industries

SECTION V — PHYSICAL HAZARDS )
Stability: Stable Conditions to Aveid: Keep away from heat or open flame
Incompatibility (Materials to Avoid): Strong oxidizing material and acids
Hazardous Decomposition Products or Byproducts: None known to Hillyard Industries
Hazardous Polymerization: Will not occur Conditions to Aveid: None known

SECTION VI ~ HEALTH HAZARD DATA
Routes of entry:  Inhalation? No Skin? No Ingestion? No
HEALTH HAZARDS (1. Acute and 2. Chronic)
1. None known to Hillyard; not an eye or skin irritant per FHSA Method 16 CFR 1500. Not acutely toxic (oral) to
laboratory rats at 5 g/kg per upper limit testing. Acute inhalation - 1 Hour Toxic Limit test - not toxic at actual
concentration of at least 14.5 mg/1.
2. Prolonged or repeated eye contact may produce redness and slight irritation.
Chemical listed as Carcinogen or Potential Carcinogen:
National Toxicology Program = No LA.R.C. Monographs =  No OSHA= No
This product has no carcinogens listed by IARC, NTP, NIOSH, or ACGIH as of this date, greater or equal to 0.1%.





[image: image2.png]page 2 of #186 Lemon Odor Counteractant

SECTION VI - HEALTH HAZARD DATA cont.

“igns and Symptoms of Exposure: None known to Hillyard Industries.

_{edical Conditions Generally Aggravated by Exposure: None known to Hillyard Industries
Emergency and First Aid Procedures: Eyes: Immediately flush with plenty of water.

SECTION VII —- PRECAUTIONS FOR SAFE HANDLING AND USE

Steps To Be Taken In Case Material Is Released Or Spilled: Absorb large spills with floor dry material. Small spills:
Flush with water and pick up with mop.

Waste Disposal Method: Dispose of in accordance with state and local regulations. Since product is an aqueous solution
containing less than 24% alcohol and with a pH of less than 12.5 and greater than 2, it would not be considered a federal
hazardous waste per EPA regulations (40 CFR 261).

Precautions To Be Taken In Handling And Storing: Keep container closed when not in use. Since empty packages retain
product residue, follow label warnings even after package is empty.

Other Precautions: Do not dispose of in storm drains or streams. This product contains no reportable quantities of toxic
chemicals subject to reporting requirements of Section 313 of SARA Title III Emergency Planning and Community Right to
Know Act of 1986 and of 40 CFR Part 372.

SECTION VIII - CONTROL MEASURES:

Respiratory Protection (Specify Type): Ventilate to keep air below TLV. For fogging use appropriate NIOSH or MSHA
approved respirator.

Ventilation: Recommend at least 5 CFM

Local Exhaust = N/A Mechanical (General) = Recommended  Special= N/A  Other= N/A
Protective Gloves:  None required Eye Protection: None required

Other Protective Clothing or Equipment: None required

Work / Hygienic Practices: Wash thoroughly after handling. Do not spray on food or foodstuffs.

£ CTION IX - TRANSPORTATION INFORMATION:
Applicable regulations: CFR = no, IMCO = yes, IATA= yes
Proper shipping name for air and foreign water: Flammable Liquid, n.o.s. (Isopropy! alcohol), 3.UN1993, il
Proper shipping name for U.S. Highway: Cleaning compound
UN No.: not applicable for highway Limited Qty.; notapplicable Hazard class: not applicable for highway
Labels required: not required for highway DOT Exception: not applicable
EPA Hazardous waste characteristics:
Ignitability = not applicable; Corrosivity = not applicable; Reactivity = not applicable.

DISCLAIMER OF WARRANTIES
NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, OF MERCHANTABILITY,
FITNESS FOR A PARTICULAR PURPOSE, OR OF ANY NATURE ARE MADE WITH RESPECT TO THE
PRODUCT(S) OR INFORMATION CONTAINED IN THIS MATERIAL SAFETY DATA SHEET.
The information and recommendations contained in this Material Safety Data Sheet are supplied pursuant to 29
CFR 1910.1200 of the Occupational Safety and Health Standards Hazard Communication Rule. All information
contained herein is presented in good faith and is believed to be appropriate and accurate.
THE BUYER OR USER ASSUMES ALL RISKS ASSOCIATED WITH THE USE, MISUSE OR DISPOSAL OF
THIS PRODUCT. THE BUYER OR USER IS RESPONSIBLE TO COMPLY WITH ALL FEDERAL, STATE OR
LOCAL REGULATIONS CONCERNING THE USE, MISUSE OR DISPOSAL OF THESE PRODUCTS.

-





